OURE

Orbital Uncertainty & Risk Engine

Satellite Collision Probability Solver

Architecture & Class Design Reference

Version 0.1.0  ·  Python 3.10+  ·  Senior Aerospace Software Engineering

# 1. Project Overview

OURE is a high-performance command-line tool for predicting the Probability of Collision (Pc) between satellites in Low Earth Orbit (LEO). It ingests real-time orbital data, propagates state uncertainties using both analytical and Monte Carlo methods, and applies Foster's algorithm on the B-plane to produce actionable risk assessments.

| Parameter | Value |
| --- | --- |
| Accuracy Target | Pc estimates within 10% of reference for miss distances 0.1–10 km |
| Performance | Screen 27,000 objects in < 30 seconds (KD-Tree O(N log N)) |
| Latency | Full Pc pipeline (single pair) < 200ms on commodity hardware |
| Languages | Python 3.10+, with sgp4, numpy, scipy, click |
| Data Sources | Space-Track.org (TLE), NOAA SWPC (F10.7 solar flux) |
| Risk Threshold | RED ≥ 1×10⁻³ ¦ YELLOW ≥ 1×10⁻⁵ ¦ GREEN < 1×10⁻⁵ |

# 2. Modular Architecture — Separation of Concerns

The system is divided into five strictly decoupled layers. Each layer speaks only through the core data models defined in oure/core/models.py. No layer imports from a layer it doesn't own.

## 2.1 Layer Map

| Module | Layer | Responsibility |
| --- | --- | --- |
| core/models.py | Data Models | StateVector, TLERecord, CovarianceMatrix, ConjunctionEvent, RiskResult — immutable frozen dataclasses. Zero business logic. Every other layer depends on this. |
| data/fetchers.py | Ingestion | SpaceTrackFetcher, NOAASolarFluxFetcher, CacheManager. Owns all I/O. The physics engine never calls these directly. |
| physics/propagator.py | Physics Engine | SGP4Propagator, J2PerturbationCorrector, AtmosphericDragCorrector, PropagatorFactory. Decorator chain for layered perturbation models. |
| uncertainty/covariance.py | Uncertainty | STMCalculator, CovariancePropagator, MonteCarloUncertaintyPropagator. Analytical STM and Monte Carlo ghost trajectories. |
| conjunction/assessor.py | Risk Math | ConjunctionAssessor (KD-Tree + TCA), RiskCalculator (B-plane + Foster's Pc). |
| cli/commands.py | Interface | Click CLI: fetch / analyze / monitor / cache subcommands. |

## 2.2 Data Flow

The analyze pipeline flows strictly top-to-bottom with no circular dependencies:

Space-Track API  ──▶  SpaceTrackFetcher  ──▶  TLERecord

NOAA API         ──▶  NOAASolarFluxFetcher ──▶ SolarFluxData

│

PropagatorFactory.build(tle, solar_flux=F10.7)

SGP4Propagator ◀── J2Corrector ◀── DragCorrector

│

CovariancePropagator.propagate(P₀, Φ, Δt)

MonteCarloUncertaintyPropagator.run(N=1000)

│

ConjunctionAssessor.find_conjunctions()  ← KD-Tree O(N log N)

│

RiskCalculator.compute_pc()  ← B-plane + Foster integral

│

RiskResult { pc, warning_level, sigma_bplane }

# 3. Physics Engine — Propagator Stack

## 3.1 SGP4 Base Propagator

SGP4 (Simplified General Perturbations 4, Hoots & Roehrich 1980) is the industry standard for TLE-based orbit prediction. The propagation pipeline follows these steps:

- Compute time since TLE epoch: tsince (minutes)

- Apply secular drag via B* term to update semi-major axis a

- Apply secular J2 terms to RAAN (Ω) and arg-of-perigee (ω)

- Update mean anomaly M = M₀ + n·Δt

- Solve Kepler's Equation via Newton-Raphson: M = E − e·sin(E)

- Convert (a, e, i, Ω, ω, ν) → ECI via PQW rotation matrix

| NOTE | In production, delegate to the sgp4 PyPI library (Vallado's port) for the certified NORAD SGP4/SDP4 formulation. The SGP4Propagator wrapper provides a clean StateVector-based interface over the library. |
| --- | --- |

## 3.2 J2 Perturbation Layer

Earth's equatorial bulge (J₂ = 1.08263 × 10⁻³) causes two secular drifts:

RAAN precession (orbital plane rotation):

dΩ/dt = −3/2 · J₂ · (R_E/p)² · n · cos(i)

At ISS altitude (410 km, i = 51.6°): dΩ/dt ≈ −7.3°/day

Argument-of-perigee drift (ellipse rotation in its plane):

dω/dt = +3/4 · J₂ · (R_E/p)² · n · (5cos²i − 1)

Sun-synchronous orbits are designed so J₂ precession matches the solar drift rate (~0.9856°/day).

The J₂ acceleration in ECI Cartesian form (used for STM Jacobian computation):

a_J2 = −3/2 · J₂ · μ · R_E² / |r|⁵ · [x(1−5z²/r²), y(1−5z²/r²), z(3−5z²/r²)]

## 3.3 Atmospheric Drag Layer

Solar activity directly modulates upper-atmosphere density via heating of the thermosphere (200–1000 km altitude). Higher F10.7 → denser atmosphere → more drag → faster orbital decay.

Density model (exponential with solar flux correction):

ρ(h) = ρ₀ · exp(−(h − h₀) / H) · exp(k_F · (F10.7 − 150))

where H is the scale height (~7–9 km in LEO) and k_F ≈ 0.003 is the Jacchia-Roberts solar coupling constant. At F10.7 = 250 (solar max), density at 400 km is ~10× the solar minimum value.

Drag deceleration:

a_drag = −½ · (C_D · A/m) · ρ · v_rel² · v̂_rel

# 4. Covariance Matrix Propagation — The Core Math

## 4.1 What the Covariance Represents

A satellite's true state is a 6D random variable x = [x, y, z, vx, vy, vz]ᵀ ∈ ℝ⁶. Our knowledge uncertainty is encoded in the 6×6 covariance matrix P:

P = E[(x − x̄)(x − x̄)ᵀ]

Diagonal entries are variances (σ²): P[0,0] = σ²_x (uncertainty in x-position, km²). Off-diagonal entries are cross-correlations: P[0,3] = Cov(x, vx).

| KEY INSIGHT | For LEO circular orbits, along-track position uncertainty grows fastest. A small velocity error changes orbital energy, which changes the period, which shifts the satellite's along-track position by kilometres per hour. This 'along-track error inflation' is the primary driver of Pc uncertainty. |
| --- | --- |

## 4.2 State Transition Matrix (STM)

The STM Φ(t, t₀) is a 6×6 matrix that linearly maps how an initial state perturbation δx₀ evolves into δx(t):

δx(t) = Φ(t, t₀) · δx₀

Φ satisfies the matrix ODE:

dΦ/dt = A(t) · Φ,    Φ(t₀, t₀) = I₆

where A(t) = ∂f/∂x is the Jacobian of the equations of motion. For a two-body orbit with J₂:

A = [ 0₃×₃   I₃×₃  ]

[ G_tot  0₃×₃  ]

G_tot = G_2body + ΔG_J2 is the full gravity gradient tensor. The STM is evaluated using the matrix exponential:

Φ(t, t₀) = exp(A · Δt)

## 4.3 Analytical Covariance Propagation

Once Φ is known, the covariance propagates exactly for linear dynamics:

P(t) = Φ(t, t₀) · P(t₀) · Φᵀ(t, t₀) + Q

Q is the process noise matrix accounting for unmodelled forces (solar radiation pressure, outgassing):

Q = diag(q·Δt³/3, q·Δt³/3, q·Δt³/3, q·Δt, q·Δt, q·Δt)

where q ≈ 10⁻¹⁰ km²/s³ for a well-tracked LEO satellite.

## 4.4 Monte Carlo Ghost Trajectory Method

For nonlinear orbits or multi-day propagations, the linear STM approximation breaks down. The Monte Carlo method is the ground truth:

- STEP 1 — Sample: Cholesky-decompose P₀ = L·Lᵀ, then draw N samples:

x_i = x̄₀ + L · ξᵢ,   ξᵢ ~ N(0, I₆)

- STEP 2 — Propagate: Run each 'ghost' satellite through the full physics stack (SGP4 + J2 + Drag)

- STEP 3 — Reconstruct: Compute sample mean and covariance from the ensemble:

P(t) = 1/(N−1) · Σᵢ (xᵢ(t) − x̄(t))(xᵢ(t) − x̄(t))ᵀ

| CONVERGENCE | N=1000 samples achieves Pc estimates within 10% of the analytical value for typical LEO conjunctions (Alfriend 2009). Use N=2000+ for high-eccentricity orbits or propagation intervals exceeding 5 orbital periods. |
| --- | --- |

## 4.5 STM Fidelity Levels

| Fidelity | Compute Time | Description |
| --- | --- | --- |
| Level 0 | ~5 µs | Two-body analytical — exact Keplerian STM via matrix exponential. Ignores all perturbations. |
| Level 1 | ~50 µs | J₂ linearised — adds first-order J₂ correction to gravity gradient tensor ΔG. Default for Pc computation. |
| Level 2 | ~500 µs | Numerical finite-difference — perturbs each state component by ε, propagates ±ε. Requires 12 propagator calls per STM. Used for high-value RED conjunctions. |

# 5. Conjunction Assessment Pipeline

## 5.1 KD-Tree Spatial Indexing

Brute-force pairwise comparison of 27,000 objects at each time step costs O(N²) ≈ 360 million operations. The KD-Tree reduces this to O(N log N) ≈ 400,000 operations — a 1000× speedup.

- Build KD-Tree over all propagated satellite positions at each time step (30-second intervals)

- Query each primary satellite's neighbourhood within the screening distance (default: 5 km)

- Collect candidate pairs: any secondary that enters the 5 km sphere at any time step

- Stage 2: run golden-section TCA refinement for each candidate pair

## 5.2 TCA Refinement — Golden-Section Search

For each flagged pair, the Time of Closest Approach (TCA) is found by minimising the scalar range function ρ(t) = |r_p(t) − r_s(t)| over the flagged time window:

t_TCA = argmin_t  |r_p(t) − r_s(t)|

Golden-section search converges in ~50 function evaluations to 0.1-second accuracy. Each evaluation requires one propagation call per object (2 total per step).

# 6. Risk Calculation — B-Plane & Foster's Algorithm

## 6.1 The B-Plane

The B-plane is a 2D plane centred on the primary satellite at TCA, oriented perpendicular to the relative velocity vector v_rel. The probability of collision is dominated by where the secondary passes in this plane.

B-plane coordinate axes (right-handed orthonormal set):

ξ̂ = (ŷ × v̂_rel) / |ŷ × v̂_rel|

ζ̂ = v̂_rel × ξ̂

The 3D combined position covariance C_3D = C_primary + C_secondary is projected to 2D via the 2×3 projection matrix T = [ξ̂, ζ̂]ᵀ:

C_2D = T · C_3D · Tᵀ    (3×3 → 2×2)

## 6.2 Foster's Algorithm

The Probability of Collision is the integral of the combined bivariate Gaussian PDF over the collision disk of radius R (combined hard-body radius):

Pc = ∬_{ξ²+ζ²≤R²}  f(ξ−b_ξ, ζ−b_ζ; C_2D)  dξ dζ

where b = T·Δr is the B-plane miss vector and f is the 2D Gaussian PDF:

f(ξ,ζ) = 1/(2π|C_2D|^½) · exp(−½ [ξ,ζ] C_2D⁻¹ [ξ,ζ]ᵀ)

OURE implements two evaluation methods:

- Numerical dblquad (default): scipy.integrate.dblquad over ±5σ box. Accurate to machine precision for well-conditioned covariances.

- Foster series (fallback): Analytical series expansion, optimal for the high-σ/R regime typical in conjunction screening (σ >> R).

| Alert Level | Pc Range | Response Action |
| --- | --- | --- |
| GREEN | Pc < 1×10⁻⁵ | Routine monitoring. No action required. |
| YELLOW | 1×10⁻⁵ – 1×10⁻³ | Elevated risk. Increase tracking cadence. Prepare contingency manoeuvre. |
| RED | Pc ≥ 1×10⁻³ | Emergency. Notify operators immediately. Initiate avoidance manoeuvre decision. |

# 7. CLI Interface Reference

OURE provides four subcommands via the click framework. Credentials are passed via environment variables to avoid shell history exposure.

### Setup

export SPACETRACK_USER=your@email.com

export SPACETRACK_PASS=yourpassword

pip install oure

### fetch — Download TLE and solar flux data

oure fetch --sat-id 25544 --sat-id 43205

oure fetch --all-leo --output tle_cache.json

oure fetch --sat-id 25544 --force-refresh

### analyze — Run full Pc pipeline

oure analyze --primary 25544 --secondary 43205

oure analyze --primary 25544 --secondaries-file leo.json \

--look-ahead 72 --mc-samples 2000

### monitor — Continuous watch with alerts

oure monitor --primary 25544 \

--secondaries-file catalog.json \

--alert-threshold 1e-4 \

--interval 1800

### cache — Manage local SQLite cache

oure cache --status

oure cache --clear-tles

oure cache --clear

# 8. Design Patterns & Engineering Decisions

| Pattern | Applied In | Rationale |
| --- | --- | --- |
| Decorator Chain | Propagator stack | SGP4 ← J2Corrector ← DragCorrector. Each layer wraps the one below and delegates, making it trivial to add new perturbation models or substitute the base propagator. |
| Anti-Corruption Layer | Data ingestion | SpaceTrackFetcher returns only TLERecord objects. The physics layer never sees raw JSON or SQL rows. Swap the data source without touching any physics code. |
| Frozen Dataclasses | Core models | StateVector and other models are frozen=True (immutable). Physics functions produce new objects rather than mutating state — eliminates aliasing bugs. |
| Strategy Pattern | STM fidelity | STMCalculator.fidelity ∈ {0,1,2} selects the computation algorithm at runtime. The CovariancePropagator is agnostic to which strategy is active. |
| Factory Method | PropagatorFactory | Encapsulates the assembly of the propagator chain. Callers never need to know about internal layering order. |
| Dependency Inversion | BasePropagator ABC | ConjunctionAssessor depends on BasePropagator, not SGP4Propagator. Any propagator implementation (analytical, numerical, test stub) satisfies the interface. |

# 9. Dependencies

| Package | Role | Notes |
| --- | --- | --- |
| sgp4 ≥ 2.22 | Orbit propagation | Vallado's certified NORAD SGP4/SDP4 Python port |
| numpy ≥ 1.24 | Linear algebra | StateVectors, matrix operations, Cholesky decomposition |
| scipy ≥ 1.10 | Numerical math | KD-Tree, dblquad integration, matrix exponential (expm) |
| click ≥ 8.1 | CLI framework | Subcommands, option parsing, colour output |
| requests ≥ 2.28 | HTTP client | Space-Track.org and NOAA API calls |
| matplotlib (opt) | Visualisation | B-plane uncertainty ellipse plots |

OURE Architecture Reference  ·  v0.1.0  ·  Python 3.10+